Jetson Orin Platform

Feature:

- Support nVidia Jetson Orin NX/NANO series

- HDMI 2.1 (NX)/1.4 (NANO)

- 2 x MISI CSI connector

- 1 x GbE & 1 x 2.5GbE (POE opt.)

- 1x M.2 Key-M & 1 x M.2 Key-E & 1 x M.2 Key-B

- 4 x 3.2 USB type A

- 1 x RS232/CAN & 1 x RS232/422/485

- Linux, Jetpeck 6.2

Specification:

| Model Name | AI-NONXS |  |  |  |
| --- | --- | --- | --- | --- |
| nVidia Module | Jetson Orin NX 16GB | Jetson Orin NX 8GB | Jetson Orin Nano 8GB | Jetson Orin Nano 4GB |
| AI Performance | 100 TOPS | 70 TOPS | 40 TOPS | 20 TOPS |
| GPU | 1024-core NVIDIA Ampere architecture GPU with 32 Tensor Cores |  | 1024-core NVIDIA Ampere architecture GPU with 32 Tensor Cores | 512-core NVIDIA Ampere architecture GPU with 16 Tensor Cores |
| CPU | 8-core Arm® Cortex®-A78AE v8.2 64-bit CPU | 6-core Arm® Cortex®-A78AE v8.2 64-bit CPU | 6-core Arm® Cortex®-A78AE v8.2 64-bit CPU |  |
| Memory | 16GB 128-bit LPDDR5 102.4GB/s | 8GB 128-bit LPDDR5 102.4GB/s | 8GB 128-bit LPDDR5 68GB/s | 4GB 64-bit LPDDR5 34GB/s |
| HDMI | 2.1 |  | 1.4 |  |
| Audio | 1 x 3.5mm Headset jack with Line-out/MIC-In 1 x digital MIC pin header 1 x speaker out pin header |  |  |  |
| MIPI Camera | 2 x 4-lane MIPI CSI 22-pin connector. |
| Ethernet | 1 x GbE & 1 x 2.5GbE (POE opt) |  |  |  |
| USB | 4 x USB 3.2 type A & 1 x Micro USB (for OS flash) |  |  |  |
| Storage | 1 x M.2 Key-M PCIe x4 Gen4 2242/2260/2280 NVMe SSD |  | 1 x M.2 Key-M PCIe x4 Gen3 2242/2260/2280 NVMe SSD |  |
| Expansion | 1 x M.2 Key-E for 2230 Wi-Fi & BT module 1 x M.2 Key-B for 3042/3052 4G LTE/5G module 1 x Nano SIM slot |  |  |  |
| Serial Port | 1 x RS232/CAN & 1 x RS232/422/485 |  |  |  |
| Expansion I/O | 1 x 14-pin header: SPI, I2C, GND, 3.3V, 5V |  |  |  |
| Others | 1 x 12-pin header: LED, 5V, UART, LATCH (AT/ATX mode switch), GND, RST, RCV, PWR |  |  |  |
| Fan | 1 x 4-pin header |  |  |  |
| RTC | 1 x 2pin header |  |  |  |
| Security (Opt.) | TPM 2.0 |  |  |  |
| Button | 1 x Power On button , 1 x Recovery button, 1 x Reset button |  |  |  |
| Power Input Connector | 1 x DC Power Jack for 12V~20V 1 x 2-pin internal power connector |  |  |  |
| Dimension | 130mm x 100mm |  |  |  |
| Operation Temp. | -20~60 ℃ |  |  |  |
| OS | Linux, Jetpack 6.2 Support Jetson Orin NX/Nano modules (Super Mode-enabled) |  |  |  |

Jetson Orin module with Super mode performance:

I/O Information: